C47 / R47 SAVE/LOAD Settings	Appnote 015				2026-01-25

Setting groups as programs

Calculator settings on the C47 and R47 can be programmed and kept as reusable quick sets. The specific settings shown here are entirely user-selectable and mainly reflect the author’s own preferences. They are not recommendations and are included only to illustrate the method used in this app note.

This note originates from a forum post dated 2024-08-02 and follows that post closely. Only the settings discussed there are described and only those settings appear in the sample program. The purpose is not to review all available settings, but to introduce a small set that many engineers tend to change and to show how such settings can be grouped, saved, and reused.

All settings are programmed in PRGM and managed using WRITEP, READP, and XPORTP. Programs are saved via I/O → WRITEP and loaded via I/O → READP. A human-readable version can be archived using I/O → XPORTP. The settings shown are typical rather than authoritative and should be adapted freely by the user.

Additional settings

A second list of settings is provided for reference only. These were not part of the original forum post and are not included in the sample program. They are noted simply as related or interesting areas for further investigation when building personal quick sets.

Sample program

The program SETT.p47 has been part of the distribution PROGRAM sample files since original discussion on the forum. Refer to the downloadable distro sample programs.

Sample program SETT.p47:

C47 Program file export: Export format version 3, C47 program version 1.

0000:  Prgm #43/43: 156 bytes / 29 steps

0001:  LBL ‘SETT’

0002:    REM ‘Setting example program, for Appnote15, 2024-08-01’

0003:    REM ‘Updated 2026-01-25’

0004:    S.EUR

0005:    RWDOT ⋅

0006:    MULT·

0007:    CPXⅉ

0008:    CF ‘CPXmul’

0009:    CF ‘CONVHP’

0010:    FDIGS 31

0011:    CF ‘FRCYC’

0012:    SF ‘IRFRAC’

0013:    SF ‘PROPFR’

0014:    SF ‘DENANY’

0015:    CF ‘DENFIX’

0016:    DMX 12

0017:    SIG 03

0018:    ENGOVR

0019:    HIDE 31

0020:    RNG 6143

0021:    SF ‘dℤ.0’

0022:    CF ‘dLrgLI’

0023:    CF ‘RPHP’

0024:    SF ‘eRPN’

0025:    SSIZE8

0026:    SF ‘CPXRES’

0027:    SF ‘SPCRES’

0028:    CF ‘1024n’

0029:  .END.

The listed typical settings for operational engineering essentials, differing from the C47 / R47 defaults are discussed below:

| Manual | Setting | Program | Description |
| --- | --- | --- | --- |
| DISP p3, F2 | EUROPE | SETEUR | Set typical European settings and change preferred details below. Various bottled configs to setup regional settings. |
| DISP gF2 | RADIX | WDOT• | Choose Radix to be “wide dot”. Also change IPART and FPART to customize number formats. |
| DISP p2 fF6 | Set/clear MULT⋅ | MULT⋅ | Choose the multiplier to be ⋅ Multiplier is used for i×2 and 2×10² Alternatively use MULT× |
| DISP p2 fF2 | Set/Clear CPXj | CPXj | Choose complex operator to be j Math way to display 𝒾 EE way to display j |
| DISP p2 fF3 | Set/Clear CPXmul | CF SYS.FL CPXmul (prgm in SYS.FL) | Choice: SF:1.+i×2.; CF:1.+2.i Set for scientific way of writing complex numbers, 1.0+i×2.0 instead of default 1.0+2.0i |
| MODE/PREF CFLG | Set/Clear CONVHP | CF SYS.FL CONVHP (prgm in SYS.FL) | Clear to have the CONV menu button to be the destination unit, example: Default is set: Clear: [km→ ¦ ←mi.]: 1 [km→] means “convert 1 km to miles”. Set: [km ←¦→ mi.]: (HP = default way) 1 [km←] means “convert 1 mile to km”. |
| DISP p1 gF4 | FDIGS 31 | FDIGS 31 | Set fraction tolerance to be 10^-31 Set to show fractions without ≈, < and > if accurate within ±1E-31. |
| DISP p1 gF5 | Clear FRCYC | CF SYS.FL FRCYC (prgm in SYS.FL) | Clear to not use the full cycle on [a b/c]. SF:cycle, CF:on/off. Set to change the [a b/c] button to cycle through all fraction combinations. |
| DISP p1 fF2 | Set IRFRAC | SF SYS.FL IRFRAC (prgm in SYS.FL) | Set to enable irrational display Set to see 2𝜋 etc. instead of 6.28… Statusbar: “⋅𝕀” |
| DISP p1 fF3 | Set PROPFR | SF SYS.FL PROPFR (prgm in SYS.FL) | Set to allow mixed fractions Clear for 5ℯ/2 instead of 2½ ℯ. Statusbar: “b/” or “ab/”. |
| DISP p1 fF5 | Set DENANY | SF SYS.FL DENANY (prgm in SYS.FL) | Fractions in full denominator range. Statusbar: “b/200” with no “x”, or ”f” DENANY DENFIX Statusbar clear (def) clear (def) × or ⋅ clear set f set clear “ “ (blank) Not active in IRFRAC mode. × or ⋅ denominator fixed but fractions simplified, for DMX=12: 1/12, 1/6, 1/4, 1/3, 5/12, 1/2, 7/12, 2/3, 3/4... f denominator fixed, for DMX=12: 1/12, 2/12, 3/12, 4/12, 5/12… “ “ any fraction with denominator 1-DMX |
| DISP p1 fF6 | Clear DENFIX | CF SYS.FL DENFIX (prgm in SYS.FL) |  |
| DISP p1 fF4 | DMX 12 | DMX 12 | Choose 12 for ½ ¼ ⅜ etc. Popular settings are 120 for ³⁵⁵⁄₁₁₃; 32 for inches etc., 180, 999, 1200. Fraction denominator maximum. Statusbar: “/max” or “/200”. |
| DISP p1 F5 | Set SIG 3 | SIG 3 | Set to SIG 3 significant digits. Pick display mode and digits: Measurements and gauge calcs: SIG 3 Electronics: UNIT 3 General engineering: ENG 4 General: ALL 3 FIX..ALL, [.d] etc. will override IRFRAC. Switch back on with [a b/c]. |
| DISP p2 gF2 | Set ENGOVR | ENGOVR (alt. prgm in SYS.FL as ALLENG) | Set to ENG for over range, choose ENG/SCI: The ALL parameter specifies where to ‘jump’, eg: ALL 4 will display 0.001 and 0.0001 ALL 3 will display 0.001 and 100.×10-6 |
| DISP p2 gF6 | HIDE 31 | HIDE 31 | ≈0 for numbers < 1E-31. Mute exponents shown for minute numbers <¦1E-31¦. Example: 1×10-32 displays as ≈0 10×10-32 displays as 1×10-31 |
| DISP p2 gF5 | RNG = default 6145 | RNG 6143 | Positive and negative exponent limit Default exponent range, ±6145 may be too high. Change to 99 to be closer to conventional calculators. |
| DISP p2 F2 | Set dℤ.0 | SF SYS.FL dℤ.0 (prgm in SYS.FL) | Set for integers and reals displayed in the same FIX…ALL notation. Integers marked with ℤ: indicating no decimals and 1000 digits |
| DISP p2 F1 | Clear LRG_LI | CF SYS.FL LRG_LI (prgm in SYS.FL) | Integer large font. Clear to see up to 42 digits of integer digits in small font |
| C47 MODE p1 fF5 R47 PREF p1 F5 | Clear RPHP | CF SYS.FL RPHP (prgm in SYS.FL) | Clear RPHP to have →R →P X/Y entry sequence same as the COMPLEX sequence. Set RPHP to have →R →P entry as on HP15C et al. |
| C47 MODE p1 fF4 R47 PREF p1 F4 | eRPN default is set | SF SYS.FL eRPN (prgm in SYS.FL) | No ENTER duplicate. Clear for classic RPN (duplicate entry) |
| C47 MODE p2 F2 R47 PREF p1 F4 | SSIZE8 default 8 | SSIZE8 | Stack size 8 (default 8) |
| C47 MODE p1 F3 R47 PREF p2 F3 | CPXRES default is set | SF SYS.FL CPXRES (prgm in SYS.FL) | Allow Complex results without errors (default) |
| C47 MODE p1 F4 R47 PREF p2 F4 | SPCRES default is set | SF SYS.FL SPCRES (prgm in SYS.FL) | Allow ∞ results without errors (default) |
| C47 HOME Prefix gF6 R47 PFX gF6 | 1024n = default is clear | CF SYS.FL 1024n (prgm in SYS.FL) | Clear ‘data’ prefixes. Decimal/binary setting for UNIT disp mode: When set, 1Mi means 1024² = 2^10 in contrast when clear, 1M means 1000² = 10^6. Set to change UNIT to binary prefixes, ranging from kibi (210×1) to pebi (210×5) |

Other selected interesting settings/buttons, not in the SETT.p47 program

| [a b/c] | -- | -- | Re-activate IRFRAC. When UNIT is set after IRFRAC, clearly UNIT de-activates IRFRAC. Clear the fractions with [.d] and set with [a b/c] |
| --- | --- | --- | --- |
| MODE/PREF CFLG | BASEHP def set | SF BASEHP (prgm in SYS.FL) | Clear to have HEX/DEC/OCT/BIN/# control the X-register only, for when you need X in a different base than other stack levels Y-T. |
| DISP F4 | dSI def 2 | dSI 1 | Change to 0 to have no special BASE screen, 1 to have one top line only, and 3 to have 3 lines of BASE screen. |
| MODE/PREF CFLG | PFX.All def clear | SF PFX.All | Set to expand the UNIT range to quecto (10-30) - quetta (1030) where the default allows SI prefixes from femto (10-15) - peta (1015). |
| C47 MODE F4 R47 PREF F3 | SDIGS def 34 | SDIGS 16 | Significant digits. Change 0-34 to increase solving and integrating speed by reducing accuracy. This influences all calculations, not only the solver. Remember to set it back. |
| C47 MODE p2 R47 PREF p2 | iLI/RL def iR iC iLI | iREAL | Set by default to iLI/RE input mode. Integers are input if no decimals are found, and reals if decimals found. iR could be selected to force REAL only input. |

Author: Jaco Mostert; Copyright (C) 2026 The C47 Development Team. Permission is granted to copy, distribute and/or modify this document under the terms of the GNU Free Documentation License, Version 1.3 or any later version published by the Free Software Foundation; with no Invariant Sections, no Front-Cover Texts, and no Back-Cover Texts. A copy of the license can be downloaded from gnu.org.